Software features — requested by

Source: Extracted from Hybrid ARC-53 Technical Offer PDF (Annex 1, 4 pages). Buyer: Latvia State Border Guard. Delivery: Lidlauks, Runtorta, Isnaudas parish, Ludza municipality, LV-5701.

Scope: Software-relevant items only — GCS, flight control, datalink, MISB metadata, payload video/geo-track software, simulator. Hardware, airframe limits, spare parts, delivery, and commercial terms excluded.

Requested by: Mexico is not named in this PDF. Every row is Latvia.

| PDF | Feature | Requested by |
| --- | --- | --- |
| §1 | HD quality video signal transmission up to a radius of 100 km (at direct radio visibility). | Latvia |
| §1 | Transmission and reception of information about the subject and objects in visible light and infrared radiation, as video and images. | Latvia |
| §1 | Appropriate adjustments can be made to the system; other types of payload elements can be installed for other long-term airborne tasks. | Latvia |
| §1 | Ground Control Station (GCS) with portable flight path tracking antenna system. | Latvia |
| §1 | Training on the unmanned aircraft system (system and/or simulator). | Latvia |
| §2 | Data terminal operating frequency band 4,400–4,940 MHz. | Latvia |
| §2 | Output power sufficient for operating distance not less than 80 km, with the possibility to be set at different limit values as necessary. | Latvia |
| §2 | Flight control system ensures fully automatic operation from take-off to landing. | Latvia |
| §2 | Possibility of take-off and landing on a moving object (e.g. on ship). | Latvia |
| §2 | Automatic route adjustment and return to base in GPS prohibited areas. | Latvia |
| §2 | Landing system provides landing of the aircraft at the marked location. | Latvia |
| §2 | Payload element data entry function for further analysis. | Latvia |
| §2 | Video signal metadata configured according to MISB STD 0601 “UAS Data Link Local Metadata Set”. | Latvia |
| §2 | Payload element information display shall display at least the minimum amount of metadata specified in MISB STD 0902.8 “Motion Imagery Sensor Minimum Metadata Set”. | Latvia |
| §2 | Possibility to configure metadata and remove metadata from the image upon necessity. | Latvia |
| §2 | Aircraft equipped with a navigation system (e.g. INS). Aircraft able to operate without GPS (follow the set route or return to the specified location). | Latvia |
| §2 | ADS-B technology transponder or equivalent flight identification device (≤ 0.1 kg). Flight identification device shall be unlockable. | Latvia |
| §3 | Payload intended for identification and documentation during day and night. | Latvia |
| §3 | Video recording system compatible with all sensors installed, with sufficient power to maintain the geo point of each completed mission. | Latvia |
| §3 | Geo track modes for EO/IR to display the observed target coordinates. | Latvia |
| §3 | Distance gauge is part of the geo point. | Latvia |
| §3 | Geo track subsystem capable of indicating the precise calculated distance to the observed target. | Latvia |
| §3 | Ability to focus on an object and track the object autonomously. | Latvia |
| §4 | GCS with flight path tracking antenna system used in performance of SBG mobile vehicle or ship, SBG tasks, FRONTEX tasks, and other State and local government tasks. | Latvia |
| §4 | GCS has capability of aircraft handing-over among GCSs. | Latvia |
| §4 | Unmanned aircraft systems for the deployment and functionality of control and communication elements during their operation. | Latvia |
| §4 | Stable and high-quality data transmission at distance that is not less than 80 km (tender offer: up to 100 km). | Latvia |
| §6 | Training of State Border Guard officials on the system or on a simulator (or in the system and simulator) so each trained official is able to independently pilot, operate, and service the unmanned aircraft system. Planned number: up to 15 individuals. | Latvia |
| §6 | Complete simulator system provided with the aircraft, allowing year-round training. | Latvia |